Physics-Informed Bayesian Optimisation for SNAr Flow Reactor

Complete System Documentation

# 1. Overview

This documentation covers physics-informed multi-objective Bayesian optimisation (MOBO) scripts for the autonomous optimisation of a continuous-flow SNAr (nucleophilic aromatic substitution) reaction system. Two optimisation algorithms are implemented:

- TSEMO — Thompson Sampling for Efficient Multi-objective Optimisation (Bradford et al., 2018)

- qNEHVI — q-Noisy Expected Hypervolume Improvement (Daulton et al., 2021)

Both are extended with a physics-informed Gaussian process (piGP) prior that encodes known chemical and reactor knowledge — specifically second-order bimolecular kinetics and approximate plug-flow reactor behaviour — without requiring fitted kinetic parameters. The prior mean parameters are learned jointly with the GP kernel hyperparameters via MAP estimation at every iteration.

An optional concentration denoiser (TSEMO only) further reduces MIR/PLS measurement noise by blending physics predictions with raw measurements before objective computation.

# 2. Reaction System

## 2.1 Chemistry

SNAr of 2,4-difluoronitrobenzene (DFNB, A) with pyrrolidine (B) in ethanol:

A + B  →  C   (ortho-substituted product, desired)

A + B  →  D   (para-substituted product, side product)

C + B  →  E   (bis-adduct, over-reaction)

D + B  →  E

## 2.2 Reactor

Capillary flow reactor: V_R = 2 mL, diameter 1 mm, approximate plug-flow behaviour (PFR). Online measurement via MIR spectrometer + PLS regression gives concentrations c₁…c₅ at the outlet (~1–3% relative noise).

## 2.3 Decision Variables

| Variable | Description | Bounds |
| --- | --- | --- |
| τ (residence time) | min | 0.25 – 2.00 |
| T (temperature) | °C | 30 – 50 |

## 2.4 Objectives (both minimised)

Y₁ = −log(STY)  =  −log(M_C · c₃ / τ_sec)     [maximise space-time yield of C]

Y₂ = −log(SEL)  =  −log(c₃ / (c₃ + c₄ + c₅)) [maximise selectivity to C]

Objectives are in log-space because STY and SEL span multiple orders of magnitude and both are strictly positive. Minimising −log(f) is equivalent to maximising f.

# 3. File Structure

## 3.1 TSEMO Files

| File | Role |
| --- | --- |
| TSEMO_OPC_2D_piGP.m | Main script — run this for real or simulated experiments |
| TSEMO_V4_1a_OPC_piGP.m | Algorithm (self-contained with all subfunctions) |
| piGP_prior_mean.m | Physics prior mean m(x¦θ) for both objectives |
| NLikelihood_piGP.m | Extended MAP likelihood (GP + prior params) |
| TrainingOfGP_piGP.m | Joint hyperparameter optimisation (DIRECT + fmincon) |
| posterior_sample_piGP.m | Spectral Thompson sampling + mean_sample_piGP |
| denoise_concentration.m | Optional: physics-informed concentration denoiser |

## 3.2 qNEHVI Files

| File | Role |
| --- | --- |
| qNEHVI_piGP.m | Single-run piGP qNEHVI (multiple noise levels) |
| qNEHVI_piGP_Multiple_Runs_...m | Batch/parallel runs varying maxeval and dataset size |

## 3.3 Old Prior Scripts (for comparison)

| File | Role |
| --- | --- |
| create_learn_from_LHC_priors.m | Linear polynomial prior from LHD data |
| create_learn_from_LHC_priors_quadratic.m | Quadratic/interaction polynomial prior from LHD data |
| TSEMO_V4_1a_OPC_with_prior_NoTic.m | TSEMO algorithm with old prior injection (has consistency bug) |
| TSEMO_Multiple_Runs_...WithPrior_Quadratic...m | Batch runs with quadratic prior |

# 4. Quick Start

## 4.1 For Real Experiments

- Connect hardware (thermostat, pumps, MIR spectrometer via OPC-UA)

- Set parameters in TSEMO_OPC_2D_piGP.m: V_R, c1_0, c2_0, bounds, opt.maxeval

- Uncomment the hardware blocks (OPC-UA + MIR timer sections)

- Comment out the simulation dummy line (f_PFR_2nd_order)

- Run TSEMO_OPC_2D_piGP.m

## 4.2 For In-Silico Testing

Run TSEMO_OPC_2D_piGP.m as-is. The simulation dummy (f_PFR_2nd_order + f_PFR_kinetics) is active by default.

## 4.3 MATLAB Path Requirements

Add to path: all piGP files listed above, plus the original TSEMO package files: invChol, paretofront, nsga2, Direct, hypervolume2D, hypervolume3D (mex files), and TSEMO_options.m.

# 5. The Physics-Informed Prior — Core Mathematics

## 5.1 Standard GP (zero prior)

μ*(x*) = K(x*, X) · K⁻¹ · y

The GP must learn the entire objective landscape from 4–10 experiments. Posterior variance is high in unexplored regions.

## 5.2 Physics-Informed GP

μ*(x*) = m̃(x* | θ) + K(x*, X) · K⁻¹ · (y − m̃(X | θ))

The prior mean m̃(x | θ) is the Damköhler-based physics prediction. The GP kernel models only the residual between physics and reality. θ = {log_kref, Ea, α, β} is learned from data at every iteration.

## 5.3 Damköhler Prior

Da(τ, T | θ) = k_ref · exp(−Eₐ/R · (1/T − 1/T_ref)) · c₀ · τ_sec

X = Da / (1 + Da)    [2nd-order equal-concentration PFR conversion]

m̃₁ = −log(α · c₀ · X · M_C / τ_sec + β)   [STY prior]

m̃₂ = −log(α / (1 + Da²) + β)              [SEL prior]

Separate {k_ref, Ea, α, β} are learned for each objective, because ortho-selectivity and bis-adduct formation have different temperature sensitivities.

# 6. Outputs

| Variable | Description | When to use |
| --- | --- | --- |
| Xpareto, Ypareto | Pareto set/front from actual experiments | Always inspect — ground truth |
| X, Y | Complete dataset (all experiments) | Post-analysis, debugging |
| XparetoGP, YparetoGP | GP-predicted Pareto set/front | Use as final result with noisy measurements |
| YparetoGPstd | GP posterior std at each Pareto point | Uncertainty quantification |
| hypf | Final kernel hyperparameters [λ₁, λ₂, σf, σn] | Diagnose GP fit quality |
| theta_history | [n_iter×4×2] prior params per iteration | Diagnose prior convergence |

# 7. References

- Bradford, E., Schweidtmann, A. M., Lapkin, A. (2018). Efficient multiobjective optimization employing Gaussian processes, spectral sampling and a genetic algorithm. J. Global Optim., 71, 407–438.

- Hone, C. A., Holmes, N., et al. (2017). Rapid multistep kinetic model generation from transient flow data. React. Chem. Eng., 2, 103–108.

- Cross, E. J. et al. (2024). A spectrum of physics-informed Gaussian processes for regression in engineering. Data-Centric Engineering, 5, e8.

- Ziatdinov, M. et al. (2022). Physics makes the difference: Bayesian optimization and active learning via augmented Gaussian process. Mach. Learn.: Sci. Technol., 3, 015003.

- Boltz, T. et al. (2025). Leveraging prior mean models for faster Bayesian optimization of particle accelerators. Scientific Reports, 15, 12232.